#!/usr/bin/env python3

"""

qcar_lane_follow_with_path_planned_avoidance_final.py

-----------------------------------------------------

✅ Final version — symmetric, correct for both sides, loop-free.

✅ If obstacle is on LEFT → move RIGHT → straight → LEFT.

✅ If obstacle is on RIGHT → move LEFT → straight → RIGHT.

"""

import time

import json

import cv2

import numpy as np

from pal.products.qcar import QCar, QCarCameras, IS_PHYSICAL_QCAR

from pal.utilities.lidar import Lidar

# =========================================================

# ---------------- Configuration --------------------------

# =========================================================

OUT_PATH = "/tmp/qcar_avoid_info.json"

STOP_DISTANCE = 0.40

FRONT_ARC_START = 45

FRONT_ARC_END = 135

NUM_MEASUREMENTS = 360

LIDAR_UPDATE_RATE = 10

# Lane following parameters

FORWARD_SPEED = 0.07

STEERING_GAIN = 0.005

STEERING_CLIP = 0.5

MIN_AREA = 500

# Path-based avoidance durations (~20 cm each)

SHIFT_TIME = .75

FORWARD_PASS_TIME =1

RETURN_TIME = 1

AVOID_SPEED = 0.07

RETURN_SPEED = 0.07

# Cooldown (to prevent repeated avoidance)

COOLDOWN_TIME = 5.0

# =========================================================

# ---------------- Steering Calibration -------------------

# =========================================================

# ✅ These have been corrected for your QCar hardware orientation.

SHIFT_STEER_RIGHT = -0.4    # steering to move RIGHT during SHIFT (left obstacle)

SHIFT_STEER_LEFT  = +0.4    # steering to move LEFT during SHIFT (right obstacle)

RETURN_STEER_RIGHT = -0.9  # steering to turn RIGHT back toward lane

RETURN_STEER_LEFT  = +0.9  # steering to turn LEFT back toward lane

# =========================================================

# ---------------- Helper Function ------------------------

# =========================================================

def write_json(data):

try:

with open(OUT_PATH, "w") as f:

json.dump(data, f)

except:

pass

# =========================================================

# ---------------- Initialization -------------------------

# =========================================================

if not IS_PHYSICAL_QCAR:

import qlabs_setup

qlabs_setup.setup()

print("✅ Starting Final QCar Path-Planned Obstacle Avoidance")

lidar = Lidar(type='RPLidar', numMeasurements=NUM_MEASUREMENTS,

rangingDistanceMode=2, interpolationMode=0)

write_json({"command": "STOP", "ts": time.time()})

cameras = QCarCameras(enableFront=True)

car = QCar(readMode=1, frequency=200)

# =========================================================

# ---------------- Main FSM -------------------------------

# =========================================================

try:

with cameras, car:

state = "LANE_FOLLOW"

avoid_cmd = "GO"

locked_avoid_cmd = None

shift_dir = None

maneuver_start = time.time()

avoid_cooldown = 0.0

lidar_period = 1.0 / LIDAR_UPDATE_RATE

last_lidar_time = 0.0

front_min = float("inf")

mean_angle = None

print("🚗 Running (Final version: symmetric, loop-free)")

while True:

t = time.time()

# ---------- LiDAR Reading ----------

if (t - last_lidar_time) >= lidar_period:

last_lidar_time = t

try:

lidar.read()

ang = np.degrees(lidar.angles)

dist = np.array(lidar.distances)

ang = (ang + 180) % 360 - 180

front_mask = (ang >= FRONT_ARC_START) & (ang <= FRONT_ARC_END)

dist_front = dist[front_mask]

valid_front = (dist_front > 0) & (dist_front < STOP_DISTANCE)

if np.any(valid_front):

close_angles = ang[front_mask][valid_front]

close_dists = dist_front[valid_front]

front_min = float(np.min(close_dists))

mean_angle = float(np.mean(close_angles))

else:

front_min = float("inf")

mean_angle = None

except:

front_min = float("inf")

mean_angle = None

# ✅ Detect obstacle only when not cooling down

if state == "LANE_FOLLOW" and (t > avoid_cooldown):

if (mean_angle is not None) and (front_min < STOP_DISTANCE):

# mean_angle > 0 → obstacle on LEFT → go RIGHT

# mean_angle < 0 → obstacle on RIGHT → go LEFT

if mean_angle > 90:

avoid_cmd = "AVOID_RIGHT"

else:

avoid_cmd = "AVOID_LEFT"

else:

avoid_cmd = "GO"

else:

avoid_cmd = "GO"  # ignore during cooldown

# ---------- Camera Lane Detection ----------

cameras.readAll()

frame = cameras.csiFront.imageData

if frame is None:

continue

hsv = cv2.cvtColor(frame, cv2.COLOR_BGR2HSV)

lower_yellow = np.array([18, 94, 140])

upper_yellow = np.array([48, 255, 255])

mask = cv2.inRange(hsv, lower_yellow, upper_yellow)

mask = cv2.GaussianBlur(mask, (7, 7), 0)

contours, _ = cv2.findContours(mask, cv2.RETR_TREE, cv2.CHAIN_APPROX_SIMPLE)

steering = 0.0

if contours:

largest = max(contours, key=cv2.contourArea)

if cv2.contourArea(largest) > MIN_AREA:

x, y, w, h = cv2.boundingRect(largest)

cx = x + w // 2

center_x = frame.shape[1] // 2

error = center_x - cx

steering = np.clip(error * STEERING_GAIN, -STEERING_CLIP, STEERING_CLIP)

# ---------- FSM Behavior ----------

forward = FORWARD_SPEED

# LANE FOLLOW

if state == "LANE_FOLLOW":

if avoid_cmd in ["AVOID_LEFT", "AVOID_RIGHT"]:

locked_avoid_cmd = avoid_cmd

shift_dir = "LEFT" if avoid_cmd == "AVOID_RIGHT" else "RIGHT"

state = "SHIFT"

maneuver_start = t

print(f"⚠ Obstacle detected: mean_angle={mean_angle:.2f}, shift_dir={shift_dir}")

# SHIFT (move sideways)

elif state == "SHIFT":

forward = AVOID_SPEED

if shift_dir == "RIGHT":

steering = SHIFT_STEER_RIGHT

elif shift_dir == "LEFT":

steering = SHIFT_STEER_LEFT

if (t - maneuver_start) > SHIFT_TIME:

state = "FORWARD_PASS"

maneuver_start = t

print(f"➡ SHIFT done ({shift_dir}) → FORWARD_PASS")

# FORWARD_PASS (go straight)

elif state == "FORWARD_PASS":

forward = AVOID_SPEED

steering = 0.0

if (t - maneuver_start) > FORWARD_PASS_TIME:

state = "RETURN"

maneuver_start = t

print(f"↩ FORWARD_PASS done → RETURN")

# RETURN (rejoin lane)

elif state == "RETURN":

forward = RETURN_SPEED

if shift_dir == "RIGHT":

steering = RETURN_STEER_LEFT

elif shift_dir == "LEFT":

steering = RETURN_STEER_RIGHT

if (t - maneuver_start) > RETURN_TIME:

state = "LANE_FOLLOW"

locked_avoid_cmd = None

shift_dir = None

avoid_cooldown = t + COOLDOWN_TIME

print(f"🏁 Avoidance complete → LANE_FOLLOW (cooldown {COOLDOWN_TIME}s)")

# ---------- Drive ----------

car.write(forward, steering, np.array([0, 0, 0, 0, 0, 0, 1, 1]))

# ---------- Display ----------

status = f"{state} | shift={shift_dir} | avoid={avoid_cmd} | mean={mean_angle if mean_angle else 'None'} | front={front_min:.2f}"

cv2.putText(frame, status, (10, frame.shape[0]-10),

cv2.FONT_HERSHEY_SIMPLEX, 0.45, (255, 255, 255), 1)

cv2.imshow("QCar Path-Planned Avoidance", frame)

cv2.imshow("Mask", mask)

print(f"[{state}] shift_dir={shift_dir}, avoid={avoid_cmd}, mean_angle={mean_angle}, steering={steering:+.2f}, front={front_min:.2f}")

if cv2.waitKey(1) & 0xFF == ord('q'):

break

except KeyboardInterrupt:

print("\n🛑 Interrupted by user.")

finally:

try:

car.write(0.0, 0.0, np.zeros(8))

lidar.terminate()

except:

pass

write_json({"command": "STOP", "ts": time.time()})

cv2.destroyAllWindows()

#!/usr/bin/env python3

"""

qcar_lane_follow_with_path_planned_avoidance_final.py

-----------------------------------------------------

✅ Final version — symmetric, correct for both sides, loop-free.

✅ If obstacle is on LEFT → move RIGHT → straight → LEFT.

✅ If obstacle is on RIGHT → move LEFT → straight → RIGHT.

"""

import time

import json

import cv2

import numpy as np

from pal.products.qcar import QCar, QCarCameras, IS_PHYSICAL_QCAR

from pal.utilities.lidar import Lidar

# =========================================================

# ---------------- Configuration --------------------------

# =========================================================

OUT_PATH = "/tmp/qcar_avoid_info.json"

STOP_DISTANCE = 0.40

FRONT_ARC_START = 45

FRONT_ARC_END = 135

NUM_MEASUREMENTS = 360

LIDAR_UPDATE_RATE = 10

# Lane following parameters

FORWARD_SPEED = 0.07

STEERING_GAIN = 0.005

STEERING_CLIP = 0.5

MIN_AREA = 500

# Path-based avoidance durations (~20 cm each)

SHIFT_TIME = .75

FORWARD_PASS_TIME =1

RETURN_TIME = 1

AVOID_SPEED = 0.07

RETURN_SPEED = 0.07

# Cooldown (to prevent repeated avoidance)

COOLDOWN_TIME = 5.0

# =========================================================

# ---------------- Steering Calibration -------------------

# =========================================================

# ✅ These have been corrected for your QCar hardware orientation.

SHIFT_STEER_RIGHT = -0.4    # steering to move RIGHT during SHIFT (left obstacle)

SHIFT_STEER_LEFT  = +0.4    # steering to move LEFT during SHIFT (right obstacle)

RETURN_STEER_RIGHT = -0.9  # steering to turn RIGHT back toward lane

RETURN_STEER_LEFT  = +0.9  # steering to turn LEFT back toward lane

# =========================================================

# ---------------- Helper Function ------------------------

# =========================================================

def write_json(data):

try:

with open(OUT_PATH, "w") as f:

json.dump(data, f)

except:

pass

# =========================================================

# ---------------- Initialization -------------------------

# =========================================================

if not IS_PHYSICAL_QCAR:

import qlabs_setup

qlabs_setup.setup()

print("✅ Starting Final QCar Path-Planned Obstacle Avoidance")

lidar = Lidar(type='RPLidar', numMeasurements=NUM_MEASUREMENTS,

rangingDistanceMode=2, interpolationMode=0)

write_json({"command": "STOP", "ts": time.time()})

cameras = QCarCameras(enableFront=True)

car = QCar(readMode=1, frequency=200)

# =========================================================

# ---------------- Main FSM -------------------------------

# =========================================================

try:

with cameras, car:

state = "LANE_FOLLOW"

avoid_cmd = "GO"

locked_avoid_cmd = None

shift_dir = None

maneuver_start = time.time()

avoid_cooldown = 0.0

lidar_period = 1.0 / LIDAR_UPDATE_RATE

last_lidar_time = 0.0

front_min = float("inf")

mean_angle = None

print("🚗 Running (Final version: symmetric, loop-free)")

while True:

t = time.time()

# ---------- LiDAR Reading ----------

if (t - last_lidar_time) >= lidar_period:

last_lidar_time = t

try:

lidar.read()

ang = np.degrees(lidar.angles)

dist = np.array(lidar.distances)

ang = (ang + 180) % 360 - 180

front_mask = (ang >= FRONT_ARC_START) & (ang <= FRONT_ARC_END)

dist_front = dist[front_mask]

valid_front = (dist_front > 0) & (dist_front < STOP_DISTANCE)

if np.any(valid_front):

close_angles = ang[front_mask][valid_front]

close_dists = dist_front[valid_front]

front_min = float(np.min(close_dists))

mean_angle = float(np.mean(close_angles))

else:

front_min = float("inf")

mean_angle = None

except:

front_min = float("inf")

mean_angle = None

# ✅ Detect obstacle only when not cooling down

if state == "LANE_FOLLOW" and (t > avoid_cooldown):

if (mean_angle is not None) and (front_min < STOP_DISTANCE):

# mean_angle > 0 → obstacle on LEFT → go RIGHT

# mean_angle < 0 → obstacle on RIGHT → go LEFT

if mean_angle > 90:

avoid_cmd = "AVOID_RIGHT"

else:

avoid_cmd = "AVOID_LEFT"

else:

avoid_cmd = "GO"

else:

avoid_cmd = "GO"  # ignore during cooldown

# ---------- Camera Lane Detection ----------

cameras.readAll()

frame = cameras.csiFront.imageData

if frame is None:

continue

hsv = cv2.cvtColor(frame, cv2.COLOR_BGR2HSV)

lower_yellow = np.array([18, 94, 140])

upper_yellow = np.array([48, 255, 255])

mask = cv2.inRange(hsv, lower_yellow, upper_yellow)

mask = cv2.GaussianBlur(mask, (7, 7), 0)

contours, _ = cv2.findContours(mask, cv2.RETR_TREE, cv2.CHAIN_APPROX_SIMPLE)

steering = 0.0

if contours:

largest = max(contours, key=cv2.contourArea)

if cv2.contourArea(largest) > MIN_AREA:

x, y, w, h = cv2.boundingRect(largest)

cx = x + w // 2

center_x = frame.shape[1] // 2

error = center_x - cx

steering = np.clip(error * STEERING_GAIN, -STEERING_CLIP, STEERING_CLIP)

# ---------- FSM Behavior ----------

forward = FORWARD_SPEED

# LANE FOLLOW

if state == "LANE_FOLLOW":

if avoid_cmd in ["AVOID_LEFT", "AVOID_RIGHT"]:

locked_avoid_cmd = avoid_cmd

shift_dir = "LEFT" if avoid_cmd == "AVOID_RIGHT" else "RIGHT"

state = "SHIFT"

maneuver_start = t

print(f"⚠ Obstacle detected: mean_angle={mean_angle:.2f}, shift_dir={shift_dir}")

# SHIFT (move sideways)

elif state == "SHIFT":

forward = AVOID_SPEED

if shift_dir == "RIGHT":

steering = SHIFT_STEER_RIGHT

elif shift_dir == "LEFT":

steering = SHIFT_STEER_LEFT

if (t - maneuver_start) > SHIFT_TIME:

state = "FORWARD_PASS"

maneuver_start = t

print(f"➡ SHIFT done ({shift_dir}) → FORWARD_PASS")

# FORWARD_PASS (go straight)

elif state == "FORWARD_PASS":

forward = AVOID_SPEED

steering = 0.0

if (t - maneuver_start) > FORWARD_PASS_TIME:

state = "RETURN"

maneuver_start = t

print(f"↩ FORWARD_PASS done → RETURN")

# RETURN (rejoin lane)

elif state == "RETURN":

forward = RETURN_SPEED

if shift_dir == "RIGHT":

steering = RETURN_STEER_LEFT

elif shift_dir == "LEFT":

steering = RETURN_STEER_RIGHT

if (t - maneuver_start) > RETURN_TIME:

state = "LANE_FOLLOW"

locked_avoid_cmd = None

shift_dir = None

avoid_cooldown = t + COOLDOWN_TIME

print(f"🏁 Avoidance complete → LANE_FOLLOW (cooldown {COOLDOWN_TIME}s)")

# ---------- Drive ----------

car.write(forward, steering, np.array([0, 0, 0, 0, 0, 0, 1, 1]))

# ---------- Display ----------

status = f"{state} | shift={shift_dir} | avoid={avoid_cmd} | mean={mean_angle if mean_angle else 'None'} | front={front_min:.2f}"

cv2.putText(frame, status, (10, frame.shape[0]-10),

cv2.FONT_HERSHEY_SIMPLEX, 0.45, (255, 255, 255), 1)

cv2.imshow("QCar Path-Planned Avoidance", frame)

cv2.imshow("Mask", mask)

print(f"[{state}] shift_dir={shift_dir}, avoid={avoid_cmd}, mean_angle={mean_angle}, steering={steering:+.2f}, front={front_min:.2f}")

if cv2.waitKey(1) & 0xFF == ord('q'):

break

except KeyboardInterrupt:

print("\n🛑 Interrupted by user.")

finally:

try:

car.write(0.0, 0.0, np.zeros(8))

lidar.terminate()

except:

pass

write_json({"command": "STOP", "ts": time.time()})

cv2.destroyAllWindows()

print("✅ QCar stopped safely.")

print("✅ QCar stopped safely.")