# Quanser QCar Deployment Guide on NVIDIA Jetson

SYSTEM TARGET

Quanser QCar hardware

NVIDIA Jetson (Nano / Xavier NX)

Quanser PAL software stack

RPLidar sensor

CSI front camera

Python control program

BEFORE YOU START

- Charge QCar battery

- Keep emergency stop accessible

- Connect Jetson via SSH or monitor

- Plug LiDAR USB

- Ensure camera ribbon is connected

- Place QCar on stand for first test

STEP 1 — Boot NVIDIA Jetson

Power ON QCar and wait 40–60 seconds.

STEP 2 — Login to Jetson

Monitor:

Username: nvidia

Password: nvidia

STEP 3 — Verify Quanser PAL Installation

python3 -c "import pal"

If error occurs, install Quanser SDK using Quanser installer.

STEP 4 — Install RPLidar Driver Support

Plug LiDAR USB then run:

ls /dev/ttyUSB*

If permission error:

sudo usermod -a -G dialout $USER

Reboot Jetson.

STEP 5 — Verify QCar Hardware Communication

python3

from pal.products.qcar import QCar

car = QCar()

car.read()

car.write(0,0,[0,0,0,0,0,0,1,1])

print("QCar communication OK")

STEP 6 — Verify Camera

python3

from pal.products.qcar import QCarCameras

with QCarCameras(enableFront=True) as cam:

cam.readAll()

print(cam.csiFront.imageData.shape)

STEP 7 — Verify LiDAR

python3

from pal.utilities.lidar import Lidar

lidar = Lidar(type='RPLidar')

lidar.read()

print(len(lidar.distances))

lidar.terminate()

STEP 8 — Transfer Script

scp qcar_lane_follow_with_path_planned_avoidance_final.py nvidia@<ip>:/home/nvidia/

STEP 9 — Make Script Executable

chmod +x qcar_lane_follow_with_path_planned_avoidance_final.py

STEP 10 — Run Script

python3 qcar_lane_follow_with_path_planned_avoidance_final.py

STEP 11 — Expected Output

Console should display startup and FSM logs.

STEP 12 — Safety Stop

Press q in window OR CTRL + C

COMMON PROBLEMS & FIXES

Camera None:

sudo systemctl restart nvargus-daemon

LiDAR Not Detected:

Check ls /dev/ttyUSB*

PAL Import Error:

source ~/quanser_pal/setup.bash

OpenCV Window Not Showing:

ssh -X nvidia@ip

STEP 15 — Performance Optimization

sudo nvpmodel -m 0

sudo jetson_clocks

COMPLETE LIBRARY LIST

System:

python3

python3-pip

python3-opencv

Python:

numpy

matplotlib (optional)

Quanser SDK:

pal.products.qcar

pal.utilities.lidar

qlabs_setup

FINAL DIRECTORY STRUCTURE

/home/nvidia/

├── qcar_lane_follow_with_path_planned_avoidance_final.py

RECOMMENDED RUN ORDER

1. Turn ON QCar

2. SSH into Jetson

3. Check LiDAR

4. Check camera daemon

5. Run script

6. Place car on track

7. Release safely

PROFESSIONAL LAB TIP

Start with FORWARD_SPEED = 0.03 and increase gradually.